Electronic Parking Disc

DIY Hobby Project

Product Specification & Technical Overview

| Document Version | 0.1 — Initial Draft |
| --- | --- |
| Date | March 2026 |
| Author | Arsalan Raghib |
| Status | Work in Progress |
| Repository | github.com/arsalanraghib/parkscheibe |

# 1. Project Overview

This document describes the design and development of a DIY Electronic Parking Disc — a self-contained, solar-assisted device that automatically displays the correct parking start time on a windshield-mounted display when a vehicle stops. The project is inspired by the OOONO P-DISC NO3 commercial product and is built entirely as a personal hobby project for learning purposes.

## 1.1 Problem Statement

In many European countries, parking zones require a physical disc showing the arrival time. Forgetting to set it — or setting it incorrectly — can result in fines. Existing manual discs rely entirely on the driver remembering to set them. This project automates the process entirely.

## 1.2 Goal

- Detect automatically when the vehicle is parked

- Display the correct parking time (rounded to next 30-minute mark)

- Require no driver interaction at all

- Operate independently — no app, no phone, no internet connection

- Run for months/years on a small battery, supplemented by solar energy

- Be compact enough to mount inside the windshield

# 2. Features & Functionality

## 2.1 Core Features

| Feature | Description | Status |
| --- | --- | --- |
| Auto-detection | Accelerometer detects when vehicle stops moving | Planned |
| Time rounding | Rounds to next 30-min interval per German parking law | Planned |
| E-Ink display | Shows time; retains image without power | Planned |
| Solar charging | Harvests daylight to extend battery life | Planned |
| Battery backup | CR2032 or LiPo for night/garage use | Planned |
| Parking indicator | Blue LED confirms parking mode is active | Planned |
| Low-battery alert | Flashes warning before battery dies | Planned |
| Magnet mount | Attaches to windshield; one-motion removal | Planned |
| No app needed | Fully standalone — no Bluetooth or Wi-Fi | Planned |

## 2.2 Operational Logic

The device follows a simple state machine:

| State | Trigger | Action |
| --- | --- | --- |
| Driving | Motion detected by accelerometer | Display off, MCU in light sleep |
| Parked | No motion for 60+ seconds | Read RTC, round time, update display, LED on |
| Low Battery | Battery < threshold | Flash red LED, display battery icon |
| Solar Charging | Light detected by solar cell | Charge battery passively |

# 3. Technical Specifications

## 3.1 Target Specifications

| Parameter | Target Value |
| --- | --- |
| Device diameter | ~80–100 mm (circular) |
| Device thickness | < 15 mm |
| Operating voltage | 3.0 V – 3.6 V |
| Battery life (no solar) | > 6 months |
| Battery life (with solar) | > 2 years (estimate) |
| Display type | E-Ink (bistable) |
| Display size | 1.54" – 2.13" |
| Parking detection time | < 90 seconds after stopping |
| Time accuracy | ± 1 minute/month (RTC) |
| Operating temperature | -20°C to +70°C |
| Mounting | Neodymium magnets |

## 3.2 Bill of Materials (Draft)

| Component | Part / Example | Purpose | Est. Cost |
| --- | --- | --- | --- |
| Microcontroller | ATtiny85 / ESP32-C3 | Main logic, sleep/wake | €2–5 |
| RTC Module | DS3231 | Accurate timekeeping | €2–4 |
| Accelerometer | ADXL345 / LIS3DH | Motion detection | €2–3 |
| E-Ink Display | Waveshare 1.54" (200×200) | Parking time display | €8–15 |
| Solar Cell | 60×60 mm 5V panel | Ambient light harvesting | €3–6 |
| Energy Harvest IC | BQ25504 | Solar charge management | €3–5 |
| Battery | LiPo 100mAh or CR2032 | Primary power source | €2–4 |
| Blue LED | 3mm blue LED + resistor | Parking mode indicator | < €1 |
| Neodymium Magnets | N52, 10×3 mm (×4) | Windshield mounting | €2–3 |
| PCB | Custom via JLCPCB | Holds all components | €5–15 |
| Enclosure | 3D-printed PLA/PETG | Housing + solar window | €3–5 |

Total estimated BOM cost: €30–65 depending on sources and quantities.

# 4. System Architecture

## 4.1 Block Diagram Description

The system is organized into four subsystems: Power, Sensing, Processing, and Output. The solar cell feeds an energy harvesting IC (BQ25504) which maintains the battery at optimal charge. The MCU spends most of its time in deep sleep, drawing microamps. The accelerometer is configured to generate a hardware interrupt when motion starts or stops, waking the MCU. On waking, the MCU reads the current time from the RTC module, rounds it to the next 30-minute boundary, and updates the E-Ink display. The display retains its image without power (bistable). A blue LED is toggled to confirm parking mode.

## 4.2 Subsystems

| Subsystem | Components | Interface |
| --- | --- | --- |
| Power | Solar cell, BQ25504, LiPo battery | Analog |
| Sensing | ADXL345 accelerometer | I2C / SPI |
| Timekeeping | DS3231 RTC | I2C |
| Processing | ATtiny85 or ESP32-C3 | Central |
| Output – Display | Waveshare E-Ink 1.54" | SPI |
| Output – Indicator | Blue LED | GPIO |

# 5. Development Plan

## 5.1 Phases

| Phase | Goal | Deliverable |
| --- | --- | --- |
| 1 – Proof of Concept | Detect motion, display rounded time via Serial | Arduino sketch + breadboard |
| 2 – Display Integration | Show time on E-Ink display | Working prototype on breadboard |
| 3 – Power Optimization | MCU deep sleep + accelerometer interrupt wake | Low-power firmware |
| 4 – Solar Integration | Add BQ25504 + solar cell to power rail | Self-sustaining prototype |
| 5 – PCB Design | Schematic + layout in KiCad | Gerber files → order from JLCPCB |
| 6 – Enclosure | 3D-printed circular housing | STL files + assembled unit |
| 7 – Testing & Iteration | Real-world car testing | Validated, refined device |

# 6. Toolchain & Project Structure

## 6.1 Software Tools

| Tool | Purpose |
| --- | --- |
| VS Code + PlatformIO | Firmware development (Arduino framework) |
| KiCad 8 | Schematic capture and PCB layout |
| draw.io (diagrams.net) | Block diagrams, state machine diagrams |
| Git + GitHub | Version control for all project files |
| FreeCAD / Fusion 360 | 3D enclosure design |
| JLCPCB / PCBWay | PCB manufacturing |

## 6.2 Repository Structure

/parking-disc-project
   README.md
   /docs
     requirements.md
     architecture.md
     bom.csv
     devlog.md
   /firmware
     /src
     /lib
   /hardware
     /schematic
     /pcb
   /mechanical
     /3d-models

# 7. Open Questions & Risks

| Item | Description | Priority |
| --- | --- | --- |
| MCU choice | ATtiny85 (simpler, lower power) vs ESP32-C3 (more capable) — decide after PoC | High |
| Display SPI timing | E-Ink refresh cycles can be slow; test update speed in parking scenario | Medium |
| CE/legal compliance | DIY device is for personal use only — not for resale without certification | High |
| Solar cell size | Must fit circular form factor; balance power harvest vs device diameter | Medium |
| Magnet strength | Magnets must hold on vibrating windshield but allow easy removal | Medium |
| Temperature range | Car interior can reach 80°C in summer — validate all components | High |

# 8. Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | March 2026 | [Your Name] | Initial draft — project kick-off |

— End of Document —